RHK-CADcam

Complete Build Specification — Version 2.1

Panel Optimiser · Tool Library · Tool Sets · Routing System · Door Builder Integration

Resident Hero Kitchen & Cabinetry  ·  June 2026

v2.1 — Updated after Room Management implementation audit

For Claude Code handoff. Read AGENTS.md and existing codebase before writing any code.

# 1. Codebase Context — Current State

This document is the complete build specification for the CNC and routing system modules of RHK-CADcam. All changes are additive — nothing requires rebuilding existing code.

v2.1 reflects a post-implementation audit of the codebase after Room Management was completed. Two corrections were made from v2.0: the Panel Optimiser entry point, and the part operations table.

## 1.1  What Is Already Built

| Canvas (F2/F3/F4) | Sidebar with Wall (F2), Cabinet (F3), Benchtop (F4) modes. Raw SVG rendering. Zustand state management. |
| --- | --- |
| Cabinet resolver | Full five-level hierarchy resolver. Parametric part generation, material resolution, joint system. |
| Benchtop Builder | Standalone module. Three-level hierarchy, polygon drawing, cutout system. |
| Door Builder | Built — app/library/doors/DoorSystemClient.tsx (737 lines). DO NOT REBUILD. |
| Room Management | Fully built. ProjectCard.tsx with expandable rooms, drag-to-sort (@dnd-kit), add/delete. RoomSwitcher.tsx in canvas sidebar. wall_dy and wall_dz on rooms table. expand param on projects page. |
| Settings | CNC Tool Setup and CNC Machine Setup tabs exist as "Coming Soon" placeholders — replace these. |
| part_operations table | Already exists with source_table, source_part_id, operation_type, tool_id, pos_x/y/z, diameter, depth, repeat_count, repeat_spacing, output_face, output_to_cnc, parameters JSONB. Read this table carefully before creating any new operations table. |
| nest_jobs / nest_sheets / nest_placements | All exist with RLS enabled. nest_jobs has project_id and cnc_machine_id. nest_sheets has sheet_number, efficiency, gcode. nest_placements has source_table, source_part_id, sheet_x/y, rotation, cut_dx/cut_dy. |
| cnc_machines / cnc_tools / cnc_machine_profiles | All exist with RLS enabled. cnc_tools has 2 rows. cnc_machine_profiles has onion_skin_z, through_cut_z, ramp_in_distance, ramp_in_feed_pct, safe_z_clearance, vacuum_zone_1-4, push_off fields. |
| All RLS | All 10 previously-flagged tables now have RLS enabled. No RLS migration required. |

## 1.2  F-Key Registry

| F2 | Wall — draw_wall mode |
| --- | --- |
| F3 | Cabinet — cabinet placement modes |
| F4 | Benchtop — draw_benchtop modes |
| F5 | Panel Optimiser — navigate to /projects/[projectId]/optimiser (not a canvas sidebar mode) |

## 1.3  Panel Optimiser — Confirmed Entry Points

Two entry points confirmed from the codebase audit:

| Production menu → Generate CNC… | The canvas menubar already has a Production menu with a disabled "Generate CNC…" item (canvasMenuConfig.ts). This is the primary entry point. Replace the disabled placeholder with a live action that navigates to /projects/[projectId]/optimiser. The projectId is available via the project prop in CanvasClient.tsx. |
| --- | --- |
| Projects page → project card | A secondary "Optimise / CNC" button or link on the project card in app/page.tsx. Navigates to /projects/[projectId]/optimiser. Placement: alongside or below the room list in the expanded card. |

⚑  The Panel Optimiser route is /projects/[projectId]/optimiser — a standalone Next.js page, not a canvas sidebar panel. It is NOT registered as F5 in the canvas sidebar. F5 is reserved for future use.

## 1.4  part_operations vs part_routes — Important

The spec v2.0 described a new part_routes table. Before creating this table, Claude Code must read the existing part_operations table which already has significant overlap. The audit found:

| part_operations.source_table | text — case_parts ¦ toekick_parts ¦ internal_parts ¦ face_zones. Exactly the source routing needed. |
| --- | --- |
| part_operations.source_part_id | uuid — FK to the part in its source table. |
| part_operations.operation_type | text — already has drill, route, pocket, saw types. |
| part_operations.tool_id | text — tool reference (currently text, not FK to cnc_tools). |
| part_operations.pos_x/y/z | numeric — operation position on the part. |
| part_operations.diameter | numeric — tool diameter for this operation. |
| part_operations.depth | numeric — operation depth. |
| part_operations.repeat_count / repeat_spacing | For drill patterns and repeated operations. |
| part_operations.output_face | top ¦ bottom ¦ front ¦ back ¦ left ¦ right — which face the operation is on. |
| part_operations.output_to_cnc | boolean — already present. |
| part_operations.parameters | jsonb — flexible overflow for any additional parameters. |

✖  Do NOT create a new part_routes table. Extend part_operations additively instead. Add: tool_id FK constraint to cnc_tools (currently text), operation_type expansion to include outline/square_off/profile/groove/raster, tool_set_id FK to cnc_tool_sets, fill_strategy, raster_angle_deg, raster_stepover_pct, offset_top/bottom/left/right_mm columns.

## 1.5  Build Order — Dependency Chain

| Step 1 | CNC Tool Library — replace Coming Soon in Settings with real tool library + shape editor |
| --- | --- |
| Step 2 | Tool Sets — new cnc_tool_sets and cnc_tool_set_operations tables + UI in Settings |
| Step 3 | Routing System — extend part_operations and door_profile_operations additively; CabinetEditModal route section |
| Step 4 | Panel Optimiser — standalone /projects/[projectId]/optimiser page |

# 2. Step 1 — CNC Tool Library

The CNC Tool Setup tab in Settings (app/settings/SettingsClient.tsx) currently renders a Coming Soon placeholder. Replace this with a full tool library and parametric shape editor.

## 2.1  cnc_tools Table — New Columns

The cnc_tools table already exists with id, name, tool_number, diameter, flutes, tool_type, material_notes, active, base_feed_rate, base_spindle_speed. Add the following via migration:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| max_depth_per_pass | numeric (mm) | null | Maximum cut depth per Z pass. If total route depth exceeds this the system auto-steps. E.g. 4mm max with 12mm deep route = 3 passes automatically. |
| cutting_length | numeric (mm) | null | Usable flute cutting length. Determines maximum single-pass depth when max_depth_per_pass is null. |
| shank_diameter | numeric (mm) | null | Shank diameter for reference only — not used in toolpath calculation. |
| shape_profile | jsonb | null | Parametric shape profile. See section 2.2. |

⚑  The existing tool_type check constraint currently allows: compression | upcut | downcut | vbit | drill | other. Expand to also include: roundover | cove | rebate | chamfer | t_slot.

## 2.2  Parametric Shape Profile

The shape_profile JSONB stores tool cross-section parameters by type:

| straight (compression/upcut/downcut) | "type":"straight","params":{"diameter":8,"cutting_length":32} |
| --- | --- |
| vbit / chamfer | "type":"vbit","params":{"diameter":12,"angle_deg":90,"tip_diameter":0.2} |
| roundover | "type":"roundover","params":{"diameter":12,"radius":4} |
| cove | "type":"cove","params":{"diameter":12,"radius":5} |
| rebate | "type":"rebate","params":{"diameter":12,"rebate_depth":6,"rebate_width":8} |
| t_slot | "type":"t_slot","params":{"shank_diameter":6,"slot_diameter":12,"slot_height":4} |
| drill | "type":"drill","params":{"diameter":5,"point_angle_deg":118} |

## 2.3  Shape Editor UI

- In CNC Tool Setup tab — alongside each tool record, a shape preview panel renders a live SVG cross-section based on shape_profile params

- Operator selects tool_type — shape editor updates to show relevant parameter inputs for that type

- Each parameter has a labelled numeric input field

- Live SVG preview renders the tool cross-section at scale, updating as values change

- SVG shows tool centreline, cutting edge profile, and shank outline

- This same SVG profile is used in Phase 2 simulator to render the tool head during animation

## 2.4  CNC Tool Setup UI

- Tool list — table showing all active tools: tool_number, name, type, diameter, max_depth_per_pass, base_feed_rate

- Add tool button — inline form or slide-out panel

- Each row expandable to show full detail including shape editor

- Inline edit — all fields editable in place, saved on blur (match existing DoorSystemClient.tsx pattern)

- Active toggle — soft delete, inactive tools hidden from selection lists

# 3. Step 2 — Tool Sets

A Tool Set is a named, ordered sequence of machining operations using specific tools, designed to achieve a defined result (e.g. "Shaker Route", "VJ Board Lines", "Beaded Profile"). Tool sets are global shop-level entities referenced by door profiles and part operations.

## 3.1  New Tables

### 3.1.1  cnc_tool_sets

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| id | uuid | gen_random_uuid() | Primary key |
| name | text | — | Display name, e.g. "Shaker Route" |
| description | text | null | Optional description |
| is_active | boolean | true | Soft delete |
| sort_order | integer | 0 | Display order in selection lists |
| created_at | timestamptz | now() | — |
| updated_at | timestamptz | now() | — |

### 3.1.2  cnc_tool_set_operations

Each row is one step in the sequence. Executed in ascending sort_order.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| id | uuid | gen_random_uuid() | Primary key |
| tool_set_id | uuid (FK) | — | FK → cnc_tool_sets.id ON DELETE CASCADE |
| sort_order | integer | — | Execution sequence. User-editable — changing it reorders the step. |
| operation_type | text | — | pocket ¦ outline ¦ square_off ¦ profile ¦ drill ¦ groove ¦ raster |
| tool_id | uuid (FK) | null | FK → cnc_tools.id |
| description | text | null | Step label, e.g. "40mm roughing pass" |
| depth_mm | numeric | null | Fixed depth in mm. Overridden by depth_eq if set. |
| depth_eq | text | null | Formula for depth, e.g. "@part.DZ * 0.4" |
| width_mm | numeric | null | Cut width in mm (pocket, groove) |
| offset_top_mm | numeric | null | Inset from top edge of route boundary (mm) |
| offset_bottom_mm | numeric | null | Inset from bottom edge |
| offset_left_mm | numeric | null | Inset from left edge |
| offset_right_mm | numeric | null | Inset from right edge |
| fill_strategy | text | null | raster ¦ spiral_in ¦ spiral_out — for pocket and raster operations |
| raster_angle_deg | numeric | 0 | Raster pass angle. 0 = horizontal, 90 = vertical. |
| raster_stepover_pct | numeric | 40 | Stepover between raster passes as % of tool diameter |
| feed_rate_override | numeric | null | Feed rate override mm/min. Null = use tool base_feed_rate × material feed_rate_pct. |
| spindle_speed_override | numeric | null | Spindle speed override RPM. Null = use tool base_spindle_speed. |
| created_at | timestamptz | now() | — |

## 3.2  Operation Types

| pocket | Area clearance. Removes all material within the route boundary down to depth. If depth exceeds tool max_depth_per_pass, auto-steps down in increments. Fill strategy: raster or spiral. |
| --- | --- |
| outline | Single perimeter pass following the exact route boundary at the specified depth. Used for finishing passes or shallow decorative grooves. |
| square_off | Corner cleanup using a small diameter tool to reach internal corners that the larger finishing bit left radiused. System calculates which corners need treatment. |
| profile | Shaped edge pass using a profiled bit (VJ, roundover, cove, chamfer, rebate). Tool follows the perimeter. Tool shape_profile determines the resulting edge geometry. |
| drill | Single point or pattern drilling. Position defined by coordinates relative to part. Supports repeat patterns (rows, grids). |
| groove | Straight line route from point A to point B. Defined by start/end coordinates relative to the part, or by edge references. Supports multi-segment paths. |
| raster | Parallel line passes across the part or defined area. Direction set by raster_angle_deg. Used for surface texturing, fluting, VJ board effects. |

## 3.3  Tool Set Builder UI

Add a "Tool Sets" tab to the CNC group in SettingsClient.tsx, placed after CNC Machine Setup:

- Tool set list on the left — name, description, active toggle, add/delete buttons

- Selected tool set detail on the right — name field, description, and the operations list

- Operations list: each step as a row with sort_order (editable number), operation_type dropdown, tool dropdown, key parameters inline

- Changing sort_order number immediately reorders the list — list always renders in ascending sort_order

- Add Operation button — appends new row with next sort_order, defaults to pocket

- Each operation row is expandable to show all parameters including fill_strategy, per-side offsets, overrides

- Delete button on each operation row

# 4. Step 3 — Routing System

## 4.1  part_operations Table — Extensions

Do NOT create a new table. Extend the existing part_operations table additively:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| tool_set_id | uuid (FK) | null | NEW — FK → cnc_tool_sets.id. When set, executes the full tool set sequence. Takes precedence over tool_id for this operation. |
| offset_top_mm | numeric | null | NEW — Inset from top edge of part (mm). Supplements existing pos_x/pos_y for boundary-based operations. |
| offset_bottom_mm | numeric | null | NEW — Inset from bottom edge |
| offset_left_mm | numeric | null | NEW — Inset from left edge |
| offset_right_mm | numeric | null | NEW — Inset from right edge |
| fill_strategy | text | null | NEW — raster ¦ spiral_in ¦ spiral_out |
| raster_angle_deg | numeric | 0 | NEW — Raster angle in degrees |
| raster_stepover_pct | numeric | 40 | NEW — Raster stepover as % of tool diameter |
| operation_type (expand) | text | — | EXPAND check constraint to include all types: drill ¦ route ¦ pocket ¦ saw ¦ outline ¦ square_off ¦ profile ¦ groove ¦ raster. Existing values remain valid. |
| tool_id (add FK) | uuid (FK) | null | ADD FK constraint to cnc_tools.id. Currently stored as text — migrate to proper FK while preserving existing data. |

⚑  part_operations already has: source_table, source_part_id, operation_type, pos_x/y/z, diameter, depth, repeat_count, repeat_spacing, output_face, output_to_cnc, parameters JSONB, is_master/master_operation_id for paired operations. All of this is retained as-is.

## 4.2  door_profile_operations Table — Extensions

The door_profile_operations table already has: profile_id, operation_type (route|drill|pocket), tool_diameter_mm, depth_mm, width_mm, offset_from_edge_mm, pass_depth_mm, feed_rate, spindle_speed, sort_order, expressions. Add:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| tool_id | uuid (FK) | null | NEW — FK → cnc_tools.id. Replaces the raw tool_diameter_mm for ATC-aware tool selection. |
| tool_set_id | uuid (FK) | null | NEW — FK → cnc_tool_sets.id. When set, executes the full tool set sequence for this door profile operation. |
| offset_top_mm | numeric | null | NEW — Per-side inset. When set, overrides offset_from_edge_mm for this edge. |
| offset_bottom_mm | numeric | null | NEW — Per-side inset from bottom edge |
| offset_left_mm | numeric | null | NEW — Per-side inset from left edge |
| offset_right_mm | numeric | null | NEW — Per-side inset from right edge |
| fill_strategy | text | null | NEW — raster ¦ spiral_in ¦ spiral_out |
| raster_angle_deg | numeric | 0 | NEW — Raster angle in degrees |
| raster_stepover_pct | numeric | 40 | NEW — Raster stepover % |
| operation_type (expand) | text | — | EXPAND check constraint: route ¦ drill ¦ pocket ¦ outline ¦ square_off ¦ profile ¦ groove ¦ raster. Existing values retained. |

⚑  offset_from_edge_mm is retained. When per-side offsets are all null, offset_from_edge_mm applies to all four sides (square inset). Existing Shaker profile data continues to work without migration.

## 4.3  DoorSystemClient.tsx Extensions

Extend the existing profile operations editor in DoorSystemClient.tsx. Additive only — no existing functionality removed:

- Expand OP_TYPES constant to include the full operation type list

- Add tool_set_id selector alongside existing tool selector — when a tool set is selected, the individual tool_id field is hidden

- Add per-side offset inputs (top/bottom/left/right) for pocket, outline, square_off operations — default to showing the single offset_from_edge_mm value in all four fields for existing operations

- Add fill_strategy selector and raster parameters when operation type is pocket or raster

## 4.4  CabinetEditModal — Part Operations Section

Add a Routes / Operations section to CabinetEditModal (app/canvas/[roomId]/CabinetEditModal.tsx). Uses the extended part_operations table.

- Part selector — dropdown showing all parts of the selected cabinet

- Operations list for the selected part — each operation as a row with name, operation_type or tool_set name, depth

- Add Operation button — presents type selector: Single Operation, Tool Set, Drill Pattern, Groove

- Single Operation — operation_type dropdown, tool selector, depth, per-side offsets, fill strategy

- Tool Set — tool_set selector, depth, per-side offsets for the route boundary

- Drill Pattern — grid editor: holes count, spacing, start offset, diameter, depth

- Groove — start/end point inputs (X/Y relative to part), width, depth, tool

- Sort order input and delete button per operation row

- output_to_cnc toggle per operation

# 5. Step 4 — Panel Optimiser

The Panel Optimiser is a standalone Next.js page at /projects/[projectId]/optimiser. Not a canvas sidebar panel. Entry points: Production menu → Generate CNC… in the canvas, and a button on the project card in app/page.tsx.

## 5.1  Entry Points — Implementation Detail

### 5.1.1  Canvas Production Menu

In canvasMenuConfig.ts, the Production menu already has { label: "Generate CNC…", disabled: true }. Change this to:

- Remove disabled: true

- Add action: () => { window.location.href = `/projects/${project.id}/optimiser` }

- The project prop is already available in CanvasClient.tsx — pass projectId through MenuActions if not already there

### 5.1.2  Projects Page

In app/page.tsx / ProjectCard.tsx, add an "Optimise" or "CNC" link within the expanded room list area — positioned below the room list and above the Add Room button. Navigates to /projects/[project.id]/optimiser.

## 5.2  Core Principles

- Temporary workspace — no writes to job tables. Parts are read as a snapshot at load time.

- Non-destructive — quantity changes and exclusions have no BOM effect

- Batch support — multiple projects share sheets for optimal material yield

- Snapshot fidelity — G-code generation saves a complete reloadable snapshot

## 5.3  Batch Project Support

Parts from batched projects are pooled per material group and nested on shared sheets. No job separation on sheets — a sheet may contain panels from multiple projects. Each part retains its job label so cut sheets identify which job each part belongs to.

### 5.3.1  nest_jobs Schema Additions

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| project_ids | uuid[] | null | NEW — Array of all project IDs in this batch. The existing project_id retains the initiating project. |
| snapshot | jsonb | {} | NEW — Complete reloadable layout snapshot saved at G-code generation time. |
| generated_at | timestamptz | null | NEW — Timestamp when G-code was generated. |

## 5.4  Six-Stage Workflow

| Stage 1 | Machine & Tool Setup — select CNC machine and cutting profile |
| --- | --- |
| Stage 2 | Project & Part Selection — batch project selection, filter by Room/Cabinet/Material, select parts |
| Stage 3 | Pre-Optimisation Settings — sheet stock (including offcuts), kerf, pad, rotation, quality slider |
| Stage 4 | Auto-Nesting — NFP + simulated annealing. Parts grouped by material+thickness. |
| Stage 5 | Manual Editing — interactive SVG canvas, drag/drop, clipboard, sheet management, undo/redo |
| Stage 6 | G-code Generation — per-sheet files, snapshot save, send to machine |

## 5.5  Filter System

Filters work as intersections. Available within Stage 2:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| Project filter | multi-select | All | Which batched projects to filter within |
| Room filter | multi-select | All rooms | Rooms from all selected projects. Prefixed with job number when multiple projects in scope. |
| Cabinet filter | multi-select | All cabinets | Updates to reflect cabinets within selected rooms |
| Material filter | multi-select | All materials | Filter to specific materials |

⚑  Filtering highlights candidates — it is a selection aid, not a hard exclude. After filtering, the operator can individually check/uncheck parts. Cut quantity can be adjusted per part (optimiser-only, no BOM effect).

## 5.6  Nesting Algorithm

No-Fit Polygon (NFP) nesting with simulated annealing optimisation. Parts grouped by material+thickness before nesting. Within each group sorted by descending area, largest placed first.

- NFP handles rectangles and arbitrary polygons (L-shapes, future curves)

- Quality slider: Fast (nearest-fit only) | Balanced (light annealing, ~5-15s) | Best (full annealing, ~30-120s)

- Rotation: 0°/90°/180°/270° tested for non-grain materials. Grain-locked materials only test grain-preserving rotations.

- New sheets opened automatically when current sheet is full

- Parts with higher nest_priority (from source part table) placed before lower-priority parts within their material group

## 5.7  Manual Editing Canvas

### 5.7.1  Layout

| Sheet canvas (main) | Current sheet — boundary, margin zones, placed parts with grain indicators and part labels. SVG rendering matching existing canvas pattern. |
| --- | --- |
| Sheet navigator (right sidebar) | All sheets listed: page number, material, efficiency %, thumbnail. Click to navigate. |
| Parts detail panel (right sidebar) | Full detail of all parts on current sheet |
| Unplaced / clipboard (bottom) | Unplaced parts and clipboard items. Quantity input for pasting multiples. Job label prefix shown for batch runs. |

### 5.7.2  Interaction

- Drag and drop — on overlap, snaps to closest valid position (no grid). Respects kerf, pad, margins, grain.

- No valid position on sheet → error: "No room on this sheet for this part."

- Copy (Ctrl+C) — copies to clipboard, original stays on sheet

- Cut (Ctrl+X) — removes to clipboard / unplaced pool

- Paste (Ctrl+V) — places at best available position near sheet centre

- Drag from unplaced pool directly onto sheet canvas

- Paste multiples — set quantity in unplaced panel, paste places N copies in sequence

- Right-click context menu: Cut, Copy, Move to Sheet N, Delete from run

- Full undo/redo — Ctrl+Z / Ctrl+Y — every action recorded

### 5.7.3  Sheet Management

- Add blank sheet — prompts: material, dimensions, optional label

- Change sheet dimensions — modal: "Re-nest this sheet? Yes / No"

- Delete sheet — all parts return to unplaced pool

- Live efficiency % per sheet, real-time

## 5.8  G-code Generation

✖  Drilling and boring always execute before routing. Non-negotiable.

✖  Inside-out sequencing always enforced — internal features before outer profiles. Non-negotiable.

### 5.8.1  Cut Sequence per Sheet

| 1 | Program header — machine init, work offset, units, safe Z |
| --- | --- |
| 2 | Sheet load pause — M00 if sheet_load_pause = true |
| 3 | Dust extraction on |
| 4 | ALL drilling/boring across ALL parts — sequenced by drill_sequence_strategy (stripe default) |
| 5 | Tool change to routing bit |
| 6 | ALL routing — inside-out order, travel-optimised between parts |
| 7 | Dust extraction off |
| 8 | Spindle off |
| 9 | Push-off if enabled |
| 10 | Program footer — home, M30 |

### 5.8.2  Drill Sequencing Strategies

| stripe (default) | Horizontal bands across sheet, left-to-right odd bands, right-to-left even (boustrophedon). Predictable. |
| --- | --- |
| nearest_neighbour | Always move to closest unvisited hole. 80-90% optimal, fast. |
| tsp | Full TSP optimisation. Best result, higher compute. Recommended for 200+ holes. |

### 5.8.3  File Naming

One G-code file per sheet:

- Single project: {JobNumber}_{MaterialCode}_{SheetNumber}.nc

- Batch run: BATCH_{Date}_{MaterialCode}_{SheetNumber}.nc

- File extension configurable per machine profile. Default .nc

## 5.9  nest_sheets Schema Additions

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| sheet_dx | numeric (mm) | — | NEW — Actual sheet width (may differ from material default for offcuts) |
| sheet_dy | numeric (mm) | — | NEW — Actual sheet height |
| is_offcut | boolean | false | NEW — True for non-standard or offcut sheets |
| sheet_label | text | null | NEW — Operator label, e.g. "Workshop offcut A" |

## 5.10  nest_placements Schema Additions

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| part_label | text | — | NEW — Snapshot of part label including job prefix for batch runs |
| project_id | uuid | null | NEW — Which project this part belongs to |
| cut_quantity | integer | 1 | NEW — Optimiser-only cut quantity |
| shape_polygon | jsonb | null | NEW — Polygon vertices snapshot for non-rectangular parts |

# 6. Machine Profile — cnc_machine_profiles Additions

The cnc_machine_profiles table already has: safe_z_clearance, onion_skin_z, through_cut_z, ramp_in_distance, ramp_in_feed_pct, vacuum_zone_1-4, push_off_enabled, push_off_distance, notes, active. All retained. Add the following via migration:

## 6.1  Identity & Axis

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| origin_corner | text | bottom_left | Sheet datum: bottom_left ¦ bottom_right ¦ top_left ¦ top_right |
| x_axis_direction | text | positive_right | positive_right ¦ positive_left |
| y_axis_direction | text | positive_up | positive_up ¦ positive_down |
| z_axis_up | boolean | true | True if positive Z is up (standard) |
| max_feed_rate | numeric | null | Machine max feed rate mm/min — safety cap |
| max_spindle_speed | numeric | null | Machine max RPM — safety cap |
| units_code | text | G21 | G21 (mm) always for this system |

## 6.2  Z Heights

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| material_surface_z | text | top_of_material | Z zero reference: top_of_material ¦ spoilboard |
| spoilboard_thickness | numeric (mm) | null | Reference spoilboard thickness for Z offset calculations |
| drill_rapid_z | numeric (mm) | 5 | Z height between drill hits — lower than safe_z for speed |

## 6.3  Tool Entry Strategy

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| entry_strategy | text | ramp | ramp ¦ helical ¦ pre_drill ¦ straight_plunge |
| ramp_in_angle | numeric (°) | 3 | Ramp descent angle. Typical 1-5°. (ramp_in_distance already exists) |
| helical_radius | numeric (mm) | 8 | Helical entry circle radius. Used when entry_strategy = helical. |
| helical_feed_pct | numeric (%) | 40 | Feed rate % during helical entry |
| helical_passes | integer | 1 | Full helical loops before reaching cut depth |
| pre_drill_tool_id | uuid (FK) | null | FK → cnc_tools.id for pre-drill entry tool |

## 6.4  Lead-in & Lead-out

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| lead_in_type | text | arc_tangent | straight ¦ arc_tangent ¦ perpendicular ¦ none |
| lead_in_length | numeric (mm) | 8 | Length or arc radius of lead-in |
| lead_in_feed_pct | numeric (%) | 60 | Feed rate % during lead-in |
| lead_out_type | text | arc_tangent | straight ¦ arc_tangent ¦ perpendicular ¦ none |
| lead_out_length | numeric (mm) | 8 | Length or arc radius of lead-out |
| lead_out_feed_pct | numeric (%) | 100 | Feed rate % during lead-out |

## 6.5  Milling Direction

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| milling_direction | text | climb | climb ¦ conventional ¦ auto |
| milling_direction_override_by_material | boolean | true | Allow materials table to override per material |

## 6.6  Pass Strategy

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| pass_strategy | text | onion_skin | single ¦ onion_skin ¦ roughing_finishing ¦ multi_depth |
| rough_pass_z_step | numeric (mm) | 8 | Max depth per roughing pass |
| finish_pass_allowance | numeric (mm) | 0.5 | Material left for finish pass |
| finish_pass_feed_pct | numeric (%) | 80 | Feed rate % for finish pass |

## 6.7  Corner Strategy

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| corner_loop_enabled | boolean | true | Small loop at external corners to maintain speed and avoid dwell marks |
| corner_loop_radius | numeric (mm) | 3 | Corner loop radius |
| dogbone_enabled | boolean | true | Dog-bone relief at internal corners for joinery |
| dogbone_radius | numeric (mm) | auto | Typically tool_diameter/2 + 0.1mm. Auto-calculated but overridable. |
| corner_dwell_ms | integer (ms) | 0 | Dwell at sharp corners when loops disabled. Default zero. |

## 6.8  Tabs

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| tabs_enabled | boolean | false | Master tab switch |
| tab_width | numeric (mm) | 6 | Tab width along cut path |
| tab_height | numeric (mm) | 4 | Tab height above spoilboard |
| tab_min_part_area | numeric (mm²) | 10000 | Min part area before tabs added (100×100mm) |
| tabs_per_side_min | integer | 1 | Minimum tabs per side |
| tabs_per_side_max | integer | 2 | Maximum tabs per side |

## 6.9  Cut Sequencing

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| drill_sequence_strategy | text | stripe | nearest_neighbour ¦ stripe ¦ tsp |
| stripe_direction | text | horizontal | horizontal ¦ vertical |
| stripe_width | numeric (mm) | 50 | Stripe band height/width |
| group_by_vacuum_zone | boolean | false | Group routing cuts by vacuum zone |
| optimise_travel_path | boolean | true | Nearest-neighbour TSP between routing operations |
| vacuum_zone_layout | jsonb | {} | Zone boundary coordinates for zone-aware sequencing |

## 6.10  Sheet Handling & G-code Structure

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| sheet_load_pause | boolean | true | M00 pause at program start for sheet loading |
| dust_extraction_enabled | boolean | true | Output M-codes for dust extraction on/off |
| program_header | text | null | Custom G-code lines at file start |
| program_footer | text | null | Custom G-code lines at file end (home + M30) |
| spindle_on_code | text | M03 | Spindle start command |
| spindle_off_code | text | M05 | Spindle stop command |
| coolant_on_code | text | M08 | Coolant/air blast on |
| coolant_off_code | text | M09 | Coolant/air blast off |
| tool_change_code | text | T{n} M06 | Tool change template. {n} = tool number. |
| work_offset_code | text | G54 | Work coordinate system offset |
| decimal_places | integer | 3 | Coordinate output decimal places |
| line_numbers_enabled | boolean | false | Prefix lines with N sequence numbers |
| line_number_increment | integer | 10 | Line number increment |

# 7. Phase 2 — Toolpath Simulator

Phase 2 — built after the Panel Optimiser is complete and verified. Specified here so Phase 1 architecture accommodates it.

## 7.1  What It Shows

- Sheet flat in viewport at scale

- Spindle head as a scaled circle (tool diameter) with the tool cross-section SVG profile from the tool library

- Rapid moves — blue dashed line, no material removal

- Drill hits — pulsing circles at each position as they execute

- Routing moves — solid cutting line, material rendered removed behind tool

- Lead-in and lead-out arcs in a distinct colour

- Corner loops and dog-bones at correct scale

- Tabs shown as small uncut sections

## 7.2  Playback Controls

- Play / Pause, Speed (1×, 5×, 20×, 100×), Scrub timeline

- Step forward / back one G-code line

- Jump to start / end

- Sheet selector for multi-sheet runs

## 7.3  Technical Approach

Parses stored G-code text from nest_sheets.gcode into a sequence of 2D moves with metadata. Animates using SVG or canvas rendering at the requested playback speed. No re-calculation — purely replays the stored record.

# 8. Implementation Notes for Claude Code

## 8.1  Read First — Mandatory

- AGENTS.md — Next.js 16.2.6 has breaking changes. Read node_modules/next/dist/docs/ before any routing or page code.

- app/canvas/[roomId]/canvasMenuConfig.ts — Production menu already has disabled Generate CNC… item. This is the optimiser entry point.

- app/canvas/[roomId]/CanvasClient.tsx — understand how project prop is available and how to pass projectId to the menu action

- app/page.tsx and app/ProjectCard.tsx — room management already implemented. Add the Optimise button here.

- app/library/doors/DoorSystemClient.tsx — existing door builder. Extend only, never rebuild.

- app/settings/SettingsClient.tsx — CNC group structure and Coming Soon placeholders.

- src/lib/types.ts — Room, Wall, Project types. Understand existing shapes before adding.

- Database table part_operations — read carefully before writing any operations-related migration. It is already substantially built.

## 8.2  Critical Constraints

✖  Panel Optimiser is at /projects/[projectId]/optimiser — NOT a canvas sidebar panel

✖  Entry point is Production menu → Generate CNC… (already exists as disabled placeholder)

✖  Do NOT create part_routes table — extend part_operations instead

✖  Do NOT rebuild the door builder — additive extension only

✖  All new tables get RLS enabled with authenticated-only policies

✖  No localStorage or sessionStorage — use Zustand state

✖  Canvas rendering uses raw SVG — do NOT introduce react-konva

## 8.3  Build Sequence with Check-in Points

| Step 1 | cnc_tools migration — add max_depth_per_pass, cutting_length, shank_diameter, shape_profile, expand tool_type check constraint. CHECK IN. |
| --- | --- |
| Step 2 | CNC Tool Setup UI in Settings — replace Coming Soon with real tool library. CHECK IN. |
| Step 3 | Tool shape editor SVG preview within tool setup. CHECK IN. |
| Step 4 | cnc_tool_sets and cnc_tool_set_operations tables — migration only. CHECK IN. |
| Step 5 | Tool Sets UI in Settings — new tab in CNC group. CHECK IN. |
| Step 6 | part_operations extensions — migration: tool_set_id, offset columns, fill_strategy, raster fields, operation_type expansion, tool_id FK. CHECK IN. |
| Step 7 | door_profile_operations extensions — migration: tool_id FK, tool_set_id, per-side offsets, fill_strategy, raster fields, operation_type expansion. CHECK IN. |
| Step 8 | DoorSystemClient.tsx extensions — new operation types, tool set selector, per-side offsets, fill strategy. CHECK IN. |
| Step 9 | CabinetEditModal Part Operations section. CHECK IN. |
| Step 10 | Panel Optimiser page route /projects/[projectId]/optimiser and Stage 1+2 UI. CHECK IN. |
| Step 11 | Stage 3 pre-optimisation settings and Stage 4 NFP nesting engine. CHECK IN. |
| Step 12 | Stage 5 manual editing SVG canvas. CHECK IN. |
| Step 13 | Stage 6 G-code generation, export, snapshot save. CHECK IN. |
| Step 14 | Enable Production menu Generate CNC… entry point and add Optimise button to ProjectCard. CHECK IN. |
| Step 15 | Batch project support — project_ids on nest_jobs, job prefix labels, batch canvas identification. CHECK IN. |
| Step 16 | Integration testing across a real job with multiple rooms and materials. CHECK IN. |
| Phase 2 | Toolpath simulator — separate sprint after Step 16 verified. |

## 8.4  Technology Stack

| Frontend | React 19, Next.js 16.2.6, Tailwind CSS v4 |
| --- | --- |
| State | Zustand v5 |
| Canvas rendering | Raw SVG — matching existing CanvasSVG.tsx pattern. Do NOT use react-konva. |
| Backend | Supabase (xivrjteialwqmleahkfq), @supabase/ssr |
| Drag and sort | @dnd-kit — already installed and used in ProjectCard.tsx |
| 3D | Three.js via @react-three/fiber — existing 3D components only |

End of Specification  ·  RHK-CADcam v2.1  ·  June 2026