RHK-CADcam

Drill Block, Drill Library & Tool Assignment

Build Specification Addendum — Anderson EVO49 Gang Drilling

Companion to Spec v2.1 · Resident Hero Kitchen & Cabinetry · June 2026

For Claude Code implementation. Read AGENTS.md and existing codebase first.

# 1. Overview & Purpose

This addendum specifies four interconnected pieces of work that together enable correct CNC drilling output for the Anderson EVO49 machine, including ganged (simultaneous multi-spindle) drilling:

| A. Drill Library | A new cnc_drills table — a shop-wide library of drill bits, separate from the router/profile tool library (cnc_tools). Drills have different properties (no tool number, no ATC, diameter-driven). |
| --- | --- |
| B. Drill Block Setup | A new graphical configuration of the physical drill block per machine — the L-shaped spindle grid (12 in X, 8 in Y for the EVO49), with each spindle individually assignable to a drill from the library. |
| C. Tool Assignment Refactor | Every operation-bearing table moves from a single loose tool column to a three-column pattern: router_tool_id (FK), drill_id (FK), auto_tool (boolean). The operation type drives which library the dropdown shows. |
| D. Gang-Detection & G-code | The boring G-code generator detects holes that can be drilled simultaneously by adjacent same-diameter spindles on the 32mm grid, and emits the Anderson M88/M89 bitmask format. |

This addendum is a companion to Spec v2.1. It does not replace anything; it adds the drill block dimension to the CNC system. Build order: this work slots in as part of Step 1 (CNC Tool Library) and Step 4 (Panel Optimiser G-code generation) of the v2.1 build sequence.

# 2. Machine Background — Anderson EVO49 Drill Block

The following facts about the target machine were established and must be encoded in the system. These were decoded from the IntelliCAM Anderson post-processor (.arp) and two sample output files (.anc).

## 2.1  Physical Block Layout

- The drill block is L-shaped: one row of 12 spindles running along the X axis, and one row of 8 spindles running along the Y axis

- Spindle no. 1 is the shared corner spindle — it belongs to BOTH the X row and the Y row (physically one spindle). Assigning a bit to X-position-1 must automatically set Y-position-1 to the same bit, and vice versa

- Spacing is 32mm between all spindles (System 32 standard) in both X and Y

- All spindles are vertical (drill down into the panel face). There are NO horizontal/edge-drilling spindles, and they will never be used — do not model horizontal drilling

- There is only one drill block on this machine. Model one block per machine; multiple blocks per machine is out of scope

## 2.2  The Bitmask Mechanism — How Ganging Is Expressed

The Anderson controller selects which spindles fire using a bitmask B-value. Each spindle has a bit value of 2^(position-1). To gang multiple spindles in one plunge, the bit values are summed.

| Spindle No. | Bit Value | Note |
| --- | --- | --- |
| 1 | 1 | 2^0 — shared corner spindle |
| 2 | 2 | 2^1 |
| 3 | 4 | 2^2 |
| 4 | 8 | 2^3 |
| 5 | 16 | 2^4 |
| 6 | 32 | 2^5 |
| 7 | 64 | 2^6 |
| 8 | 128 | 2^7 — last Y spindle |
| 9 | 256 | 2^8 |
| … | … | continues for X spindles up to 12 |
| 12 | 2048 | 2^11 — last X spindle |

Worked example from the sample output (M88 bank): MASTER spindle 5, SLAVES 6,7,8,9 produced B496.

spindle 5 = 16 spindle 6 = 32 spindle 7 = 64 spindle 8 = 128 spindle 9 = 256 ---- B value = 496 (16+32+64+128+256)

Worked example from the second sample (M89 bank): MASTER spindle 3, SLAVES 4,5,6,7 produced B124.

spindle 3 = 4 spindle 4 = 8 spindle 5 = 16 spindle 6 = 32 spindle 7 = 64 ---- B value = 124 (4+8+16+32+64)

## 2.3  M88 vs M89 — Two Banks, Independent Bit Spaces

| M88 | The X bank — spindles 1 through 12 running along the X axis. Bit values 2^(N-1) for N=1..12. |
| --- | --- |
| M89 | The Y bank — spindles 1 through 8 running along the Y axis. Bit values 2^(N-1) for N=1..8. This is an INDEPENDENT bit-value number space addressed via M89 rather than M88. |
| M88 B0 | Clears/deselects the X bank. |
| M89 B0 | Clears/deselects the Y bank. |
| Shared spindle 1 | Belongs to both banks. In M88 it is bit 1 of the X space; in M89 it is bit 1 of the Y space. Physically the same drill. |

⚑  Because the bit spaces are independent per bank, the gang-detection algorithm computes M88 B-values and M89 B-values separately. A given drilling line uses one bank or the other depending on whether the holes run along X (use M88) or along Y (use M89).

# 3. Anderson G-code Drilling Format — Decoded

The boring section of the generated .nc/.anc file follows this exact structure. This was decoded from the sample files and the post-processor and must be reproduced by the G-code generator.

## 3.1  Annotated Drilling Block

(DRILL TOOL: V 5MM) ; comment: tool description (DRILL MASTER: 5, SLAVE: 6,7,8,9) ; comment: which spindles gang (DIAMETER: 5.) ; comment: hole diameter M88 B0 ; clear X bank M89 B0 ; clear Y bank M23 ; (machine prep code from post) M21 ; (machine prep code from post) M89 B124 ; SELECT ganged spindles (Y bank here) G90 G0 G54 G43 H1 X237.5 Y-382.75 Z36.5 ; rapid to first master pos. ; G54 = drill block work offset G98 G81 Z4. R36.5 F3000. ; G81 drill cycle: depth Z4, ; retract R36.5, feed 3000 X64. Y-382.75 Z4. ; next ganged plunge (line bored) G80 ; cancel drill cycle G17 G91 G28 Z0 M95 ; retract / home Z M88 B0 ; clear banks M89 B0 M22

## 3.2  Key G-code Elements

| G54 | Work coordinate offset for the DRILL BLOCK. The block sits at a fixed physical offset from machine origin. This offset is read off the machine (stored in the machine profile as drill_block_work_offset, default G54). |
| --- | --- |
| G59 | Work coordinate offset for the ROUTER spindle (separate from the drill block). Routing moves use G59. |
| G52 X0 Y0 | Cancels work offsets between operations. |
| G81 | Standard drill canned cycle. Z = hole depth, R = retract plane. |
| G98 | Return to initial plane after each drill (vs G99 return to R plane). |
| G80 | Cancel canned cycle. |
| Master coordinate | The X/Y position emitted is the MASTER spindle position. The slave spindles fire automatically at their fixed 32mm offsets from the master. The post does not emit slave coordinates. |

## 3.3  Coordinate System Specifics (from post-processor)

- SIGN_Y = -1 — the Y axis is negated. All Y values in output are negative (e.g. Y-256, Y-382.75)

- TRANS_Y_COORD = 1 — Y is mirrored. Material is loaded face DOWN (confirmed by "LOAD: Material face DOWN" in the .anc header)

- The drill block carries a fixed head offset (the post shows a commented G52 X0.00 Y-128.00 "Compensation for multidrill head" and adds 128 in Y offset calculations). The exact value is read off the machine — store it as a configurable profile field

- Drilling Z uses the drill_rapid_z retract plane (R36.5 in samples) which is lower than the routing safe Z, for speed between hits

# 4. Part A — Drill Library (cnc_drills)

A new shop-wide table, separate from cnc_tools. Drills are referenced by drill block spindles and by drill operations. They are NOT called by ATC tool-change commands.

## 4.1  Rationale for a Separate Table

Drills differ fundamentally from router/profile bits: they have no ATC tool number, no per-material feed scaling in the same sense, and they live permanently in block spindles rather than being swapped into the routing spindle. A router bit is never fitted to a block spindle and vice versa. Keeping two clean tables avoids conditional/empty columns and matches physical reality.

## 4.2  cnc_drills Schema

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| id | uuid | gen_random_uuid() | Primary key |
| name | text | — | Display name, e.g. "5mm Through Drill", "8mm Dowel", "15mm Hinge Cup" |
| diameter | numeric (mm) | — | Bit diameter. The KEY field for gang-detection — only same-diameter adjacent spindles can gang |
| drill_type | text | through | through ¦ blind ¦ countersink |
| point_angle | numeric (°) | 118 | Drill point angle for reference |
| max_depth | numeric (mm) | null | Maximum plunge depth |
| total_length | numeric (mm) | null | Total bit length, for reference |
| rotation | text | clockwise | clockwise ¦ anticlockwise. Some block spindles are reverse-threaded |
| supplier_code | text | null | Optional supplier/part reference |
| notes | text | null | Free text |
| is_active | boolean | true | Soft delete flag |
| created_at | timestamptz | now() | — |
| updated_at | timestamptz | now() | — |

## 4.3  Drill Library UI

Add a Drill Library section under the Tools area in Settings, visually grouped alongside the CNC Tool (router bit) library but as a distinct table and list. Both are tool libraries; they sit next to each other (e.g. "Router Tools" and "Drill Library" sub-tabs or panels within the CNC group).

- Drill list — table showing name, diameter, drill_type, point_angle, max_depth, active toggle

- Add drill button — inline form or slide-out panel

- Inline edit, saved on blur, matching the existing DoorSystemClient.tsx pattern

- Active toggle — inactive drills hidden from spindle assignment and operation dropdowns

# 5. Part B — Drill Block Setup (Graphical)

A new graphical configuration per machine showing the physical spindle layout, with each spindle individually assignable to a drill from the library. This is essential for gang-detection: the system can only gang spindles that physically hold the same-diameter bit.

## 5.1  New Tables

### 5.1.1  cnc_drill_blocks

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| id | uuid | gen_random_uuid() | Primary key |
| cnc_machine_id | uuid (FK) | — | FK → cnc_machines.id. One block per machine |
| name | text | — | Display name, e.g. "EVO49 Main Block" |
| bank_x_count | integer | 12 | Number of spindles in the X bank (M88) |
| bank_y_count | integer | 8 | Number of spindles in the Y bank (M89) |
| spacing_mm | numeric (mm) | 32 | Spindle spacing — defaults to 32, editable for future machines |
| x_bank_mcode | text | M88 | G-code that selects the X bank |
| y_bank_mcode | text | M89 | G-code that selects the Y bank |
| shared_corner | boolean | true | True if spindle 1 is shared between both banks (EVO49 = true) |
| head_offset_x_mm | numeric (mm) | 0 | Physical block head offset in X from machine origin (read off machine) |
| head_offset_y_mm | numeric (mm) | -128 | Physical block head offset in Y (the "multidrill head compensation" — read off machine) |
| work_offset_code | text | G54 | G-code work offset for the drill block |
| is_active | boolean | true | Soft delete |
| created_at | timestamptz | now() | — |
| updated_at | timestamptz | now() | — |

### 5.1.2  cnc_drill_block_spindles

One row per physical spindle. The shared corner spindle (position 1) appears once per bank but the UI keeps them synchronised.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| id | uuid | gen_random_uuid() | Primary key |
| drill_block_id | uuid (FK) | — | FK → cnc_drill_blocks.id ON DELETE CASCADE |
| bank | text | — | x ¦ y — which bank this spindle belongs to |
| position | integer | — | Spindle position within the bank, 1-based (1..12 for X, 1..8 for Y) |
| bit_value | integer | — | The bitmask value = 2^(position-1). Stored for clarity though derivable |
| drill_id | uuid (FK) | null | FK → cnc_drills.id. The bit fitted to this spindle. Null = empty/no bit fitted |
| created_at | timestamptz | now() | — |

⚑  When shared_corner is true, the row for bank=x position=1 and bank=y position=1 represent the same physical spindle. Assigning a drill to one must update the other. Enforce this in the UI layer (and optionally a trigger).

## 5.2  Graphical Drill Block Setup UI

Lives within the CNC Machine Setup area in Settings, as a section on the selected machine. The flow:

- Operator enters the X bank spindle count (default 12) and Y bank spindle count (default 8)

- A graphical viewer draws the L-shaped block to scale: a horizontal row of bank_x_count spindles and a vertical row of bank_y_count spindles meeting at the shared corner (spindle 1), at spacing_mm centres

- Each spindle is rendered as a clickable circle showing its position number and bit value

- Clicking a spindle opens a dropdown to assign a drill from the cnc_drills library (or leave empty)

- Each spindle displays the assigned drill diameter, colour-coded by diameter so gangable runs are visible at a glance

- The shared corner spindle is visually marked and stays synchronised between banks

The viewer is SVG (matching the existing canvas/shape-editor pattern — do not introduce a new rendering library). Empty spindles are allowed and shown in a muted/outline style.

## 5.3  Why Per-Spindle Diameter Matters

Gang-detection can only fire adjacent spindles that hold the SAME diameter bit. When generating a row of 5mm shelf-pin holes, the algorithm reads the block config, finds which consecutive spindles hold 5mm bits, and only gangs those. If spindle 6 holds an 8mm bit, the gang breaks at that point. This is the entire reason the block setup must be explicit and per-spindle.

# 6. Part C — Tool Assignment Refactor

Splitting drills and router bits into two libraries means every operation that assigns a tool must choose from the correct library. The operation type drives which library is relevant.

## 6.1  The Three-Column Pattern

Every operation-bearing table replaces its single loose tool column with three columns:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| router_tool_id | uuid (FK) | null | FK → cnc_tools.id. Populated when the operation is a routing type (pocket, outline, profile, groove, raster, route) |
| drill_id | uuid (FK) | null | FK → cnc_drills.id. Populated when the operation is a drill type |
| auto_tool | boolean | false | When true, both tool columns are null and the generator selects the tool at G-code time. See section 6.4 |

✖  Exactly one of router_tool_id / drill_id is populated per operation (or neither, when auto_tool = true). The operation_type determines which one applies.

## 6.2  Operation-Type → Library Mapping

| drill | Tool comes from cnc_drills (drill_id). Dropdown shows the drill library. |
| --- | --- |
| pocket / outline / square_off / profile / groove / raster / route / saw | Tool comes from cnc_tools (router_tool_id). Dropdown shows the router bit library. |

## 6.3  Tables to Migrate

The following five operation tables must be migrated. Existing sample data may be replaced (confirmed acceptable). Each migration drops the old loose tool column(s) and adds the three new columns.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| part_operations | tool_id (text) → drop | 0 rows | Replace tool_id text with router_tool_id, drill_id, auto_tool. Already covered in v2.1 Step 6 — fold this three-column pattern into that migration. |
| joint_type_operations | tool (text), tool_diameter_mm → keep diameter, replace tool | 12 rows | Add router_tool_id, drill_id, auto_tool. The 12 rows have loose text tool labels — set those rows to auto_tool = true (preserve old label in notes). tool_diameter_mm may be retained for reference. |
| joint_instance_operations | tool_diameter_mm only | 0 rows | Add router_tool_id, drill_id, auto_tool. No data to preserve. |
| door_profile_operations | tool_diameter_mm, (v2.1 adds tool_id, tool_set_id) | 3 rows | Align with the three-column pattern: ensure tool_id from v2.1 is specifically router_tool_id, and add drill_id + auto_tool. 3 existing rows are profile routes — set router_tool_id or auto_tool. |
| drawer_slide_operations | tool (text), tool_diameter_mm | 16 rows | Add router_tool_id, drill_id, auto_tool. 16 rows have loose text labels and machine_operation (drill/route/pocket) — map drill ops to auto_tool drill, others to auto_tool router, preserving old label in notes. |

✓  The joints tables (joint_type_operations, joint_instance_operations) are NOT read by the resolver for tool data, and joint_instances has 0 rows. Migrating their tool columns has NO effect on the resolver or joint geometry. The resolver reads joint placement/geometry only; tool assignment is consumed solely at G-code generation time, which does not exist yet.

✖  Do NOT confuse joint operation tables with part_operations — they are entirely separate systems. Changing one cannot affect the other.

## 6.4  The Tool Dropdown UI

Wherever an operation assigns a tool (joint operations editor, part operations in CabinetEditModal, tool set operations, door profile operations), the tool selector behaves as follows:

- The operation type is selected first

- The tool dropdown then populates from the correct library automatically — drills for a drill operation, router bits for a routing operation

- The first option is always "Auto-select" (sets auto_tool = true, both FK columns null)

- Below a divider, the filtered tool list from the relevant library

Tool: [ Auto-select v ] -------------------------- Auto-select -------------------------- (drill op -> drills from cnc_drills) (route op -> bits from cnc_tools)

## 6.5  Auto-Select Resolution Logic

When auto_tool = true, the G-code generator resolves the tool at generation time:

- Drill operations — pick the cnc_drills row whose diameter matches the operation hole diameter. For ganged holes, also verify that diameter is actually fitted to spindles in the drill block config; if not, fall back to single-spindle drilling or flag an error

- Routing operations — pick based on need: a pocket picks the largest-diameter router bit that fits the pocket and respects internal corner radii; a profile picks the bit whose shape_profile matches the required edge; an outline picks the material default cutting tool (materials.cnc_tool_id)

- Manual selection always overrides auto-select

# 7. Part D — Gang-Detection Algorithm & G-code Output

The boring section of each sheet program must detect which holes can be drilled simultaneously and emit the M88/M89 bitmask format. This runs as part of the drilling pass (before routing) in the Panel Optimiser G-code generator.

## 7.1  Which Holes Are Gang Candidates

| Gangable | Adjustable shelf-pin holes, hinge-plate holes, and sometimes drawer-front holes — holes of the SAME diameter sitting on the 32mm grid in a row (along X) or column (along Y). |
| --- | --- |
| Never ganged | Construction holes that sit OFF the 32mm grid. These always drill individually (single spindle, single bit value). Most sheets contain a mix of both. |

## 7.2  Algorithm

For each sheet, after parts are placed, collect all drill operations and process them as follows:

- Group drill hits by diameter (only same-diameter holes can share a gang, since they need the same bit)

- Within each diameter group, identify collinear runs of holes at exact 32mm-multiple spacing — these are gang candidates. Separate runs along X (→ M88 bank) from runs along Y (→ M89 bank)

- For each run, confirm the drill block actually has consecutive same-diameter spindles available in the relevant bank. The number of gangable holes per plunge is capped by the bank size (12 for X, 8 for Y) AND by how many consecutive spindles hold that diameter

- Step-and-repeat for runs longer than the available gang: fire a gang covering the first N holes, step the master position along the run, fire again for the next N, until the run is consumed. Remainder holes that do not fill a full gang are fired as a smaller gang or individually

- Compute the bitmask: sum 2^(position-1) for the consecutive spindles used. Emit the bank M-code (M88 or M89) with B<sum>, then the master coordinate(s)

- Off-grid construction holes and any holes not matching a gang are emitted as individual drill hits (single-spindle bit value, or the appropriate single bit)

- Order the drilling pass using the drill_sequence_strategy from the machine profile (stripe default) so the head works methodically across/up the sheet rather than jumping around

## 7.3  Step-and-Repeat Example

A vertical column of 20 shelf-pin holes at 32mm centres, using the Y bank (8 spindles):

Holes 1-8 -> M89 B255 (2^0..2^7 = 255), master at hole 1 Holes 9-16 -> M89 B255, master stepped to hole 9 Holes 17-20 -> M89 B15 (2^0..2^3 = 15), master at hole 17 (4 remaining holes = spindles 1-4 only)

⚑  B255 = all 8 Y-bank spindles. B15 = first 4 spindles (1+2+4+8). The master coordinate moves to the first hole of each group; slaves fire at their fixed 32mm offsets.

## 7.4  G-code Emission Order (within the drilling pass)

Recall from v2.1: ALL drilling executes before ANY routing (non-negotiable). Within the drilling pass:

- Clear both banks (M88 B0 / M89 B0) and run any machine prep codes

- For each diameter / bit, emit the comment block (DRILL TOOL / MASTER+SLAVE / DIAMETER)

- Select the bank+bitmask, rapid to the master position with the G54 drill-block work offset, run the G81 cycle, emit subsequent master positions, then G80

- Sequence diameters to minimise bit changes and head travel, honouring drill_sequence_strategy

- Clear banks at the end of the drilling pass before the tool change to the router

✖  All Y coordinates are emitted negative (SIGN_Y = -1) and the block head offset (head_offset_y_mm, e.g. -128) is applied. Match the sample .anc output exactly.

# 8. Machine Profile Additions

The cnc_machine_profiles table (already being extended in v2.1) needs a few more drill-block-specific fields. These complement the drill block tables — the block geometry lives in cnc_drill_blocks; these are the G-code emission settings.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| drill_block_work_offset | text | G54 | Work offset code for drilling moves (router uses a separate offset, e.g. G59) |
| router_work_offset | text | G59 | Work offset code for routing moves |
| drill_rapid_z | numeric (mm) | 5 | Retract plane between drill hits (already in v2.1 — referenced here for the R value in G81) |
| drill_cycle_code | text | G81 | Drill canned cycle code |
| drill_return_code | text | G98 | Return mode after each drill (G98 initial plane / G99 R plane) |
| sign_y | integer | -1 | Y axis sign. -1 for the EVO49 (Y output negative) |
| mirror_y | boolean | true | Whether Y is mirrored (material loaded face down) |
| drill_bank_prep_codes | text | M23\nM21 | Machine prep G-code lines emitted before bank selection (from the post). Free text, machine-specific |

# 9. Implementation Notes for Claude Code

## 9.1  Read First

- AGENTS.md — Next.js 16.2.6 breaking changes. Read node_modules/next/dist/docs/ before any routing/page code

- app/settings/SettingsClient.tsx — CNC group structure; where the drill library, drill block setup, and tool library live

- The existing ShapeEditor / canvas SVG components — reuse the SVG rendering pattern for the drill block viewer

- Database tables joint_type_operations, joint_instance_operations, part_operations, door_profile_operations, drawer_slide_operations — inspect current tool columns before migrating

- The resolver in src/lib/resolver/ — confirm it does not read operation tool columns (it does not, but verify before migrating)

## 9.2  Critical Constraints

✖  Do NOT model horizontal/edge drilling — vertical spindles only

✖  Spindle 1 is shared between X and Y banks — keep synchronised in the UI

✖  Gang only same-diameter holes on the 32mm grid; construction holes always drill individually

✖  All drilling executes before all routing (carried from v2.1)

✖  Drill block viewer uses SVG — do not introduce a new rendering library

✖  Migrating joint operation tool columns does NOT affect the resolver or joint geometry — but do not touch joint placement/geometry columns

## 9.3  Build Sequence with Check-in Points

| Step 1 | cnc_drills table migration + Drill Library UI in Settings. CHECK IN. |
| --- | --- |
| Step 2 | cnc_drill_blocks and cnc_drill_block_spindles tables migration. Seed the EVO49 block (12 X, 8 Y, 32mm, shared corner). CHECK IN. |
| Step 3 | Graphical Drill Block Setup UI in CNC Machine Setup — SVG L-shaped viewer, per-spindle drill assignment, shared-corner sync, diameter colour-coding. CHECK IN. |
| Step 4 | Tool assignment refactor migration — add router_tool_id / drill_id / auto_tool to all five operation tables; drop old loose tool columns; set existing rows to auto_tool with old labels preserved in notes. CHECK IN. |
| Step 5 | Tool dropdown UI update across all operation editors — operation-type-driven library filtering + Auto-select option. CHECK IN. |
| Step 6 | cnc_machine_profiles drill-block field additions migration. CHECK IN. |
| Step 7 | Gang-detection algorithm + Anderson M88/M89 bitmask G-code emission, integrated into the Panel Optimiser drilling pass. Validate output against the two provided sample .anc files. CHECK IN. |
| Step 8 | Auto-select resolution logic for drills (diameter match against block config) and router bits. CHECK IN. |
| Step 9 | End-to-end test: a sheet with ganged shelf holes (Y bank), ganged hinge-plate holes, and off-grid construction holes — confirm output matches Anderson format and the gangs/bitmasks are correct. CHECK IN. |

## 9.4  Validation Against Samples

Two sample files were provided and must be used as golden-output references for the G-code generator:

- drillgangexample.anc — M88 bank gang, MASTER 5 SLAVE 6,7,8,9, B496

- R100101N.anc — M89 bank gang, MASTER 3 SLAVE 4,5,6,7, B124, followed by an outline route (G59 offset)

The generator output for an equivalent input must reproduce the bank selection, bitmask, G54/G59 offsets, G81/G98/G80 cycle structure, negative Y coordinates, and comment blocks shown in these files.

End of Addendum  ·  Drill Block, Drill Library & Tool Assignment  ·  June 2026